| Component | Est. Cost (CAD) for 1 Pair | Engineering Notes (Pros & Cons) | Purchase Links |
| --- | --- | --- | --- |
| CDHPOWER 29"/700C Mag Wheelset | $155.99 | Pros: Outstanding budget option. Aluminum mag construction is rust-proof and eliminates wire spoke truing entirely. 150 kg load capacity. Cons: High weight (5.9 kg) increases rotational inertia, potentially challenging the stepper motor's soft-start ramp. | Amazon Link |
| MESOVFCA 700C 5-Spoke Mag Wheel Set | $414.18 | Pros: Monocoque magnesium alloy rim design offers exceptional durability. Cons: expensive. Weighs approx 3.7 kg, impacting rotational inertia. | Amazon Link |
| DHNCBGFZ 700C 3-Spoke Mag Wheel Set | $356.73 | Pros: Magnesium alloy offers high dimensional integrity and excellent rust resistance. Cons: Expensive for a magnesium wheel; heavier than carbon fiber options at this price point. | Amazon Link |
| TANMEI Mountain Bike Magnesium Alloy 3-Spoke 29" | $333.99 | Pros: Magnesium alloy is rust-resistant. The 3-spoke design eliminates truing maintenance. Highly durable mountain bike construction easily handles the 270N static radial load. Cons: Heavy, typical of mag wheels, which increases rotational inertia. | Amazon Link |
| KHE Fixie 700c 28" Industrial Bearing Wheels | $237.90 (Front $104.95 + Rear $132.95) | Pros: Aluminum double chamber rims with industrial bearings provide excellent moisture resistance. Noticeably lighter than magnesium wheels, easing motor load. Cons: Uses traditional tension spokes, which may require occasional truing due to continuous 1 RPM rotation under shifting water bath loads. | Front Link Rear Link |
| Superteam 50mm Carbon Wheelset 700c (S09) | $329.04 | Pros: Carbon fiber construction is 100% rust-proof and highly resistant to nutrient salts. Extremely lightweight (1,630g), significantly reducing rotational inertia and easing the load on the NEMA23 stepper motor. Cons: More expensive than aluminum spoked alternatives. | Amazon Link |
| Sunrise Bike 88mm Carbon Wheelset 700c | $356.61 | Pros: Deep 88mm carbon rim provides exceptional, broad structural rigidity for mounting the 2040 extrusion plant rows. Carbon fiber is completely immune to the high-humidity aeroponic environment. Cons: Slightly heavier for a carbon wheel (1,910g) due to the massive deep rim. | Amazon Link |
| SUPERTEAM S50 Carbon Wheel 700C | $407.20 | Pros: Premium carbon fiber construction (1,630g) minimizes motor stall risk. Completely impervious to rust and galvanic corrosion. Cons: Eats up a large portion of the budget. | Amazon Link |

Plant pots

| The HC Companies Venetian Window Planter Box w/Drain Plug - Long Planter Box for Windowsills, Balconies & More - Plastic Large Rectangular Outdoor Planters for Herbs & Flowers (30" Black, Pack of 2) | $59.55 | Amazon Link |
| --- | --- | --- |
| 5-Pack 1020 Garden Growing Trays Without Drain Holes - 10" x 20" Plastic Plant Trays No Holes - Wheatgrass Sprouting Tray, Microgreens Seed Tray, Hydroponic Trays, Nursery Flats, Greenhouse Supplies | $66.35 | Amazon Link |
| Outland Living Rectangular Plastic Planters 26.5 Inch 4 Pack Forest Green - Window Planter Box for Outdoor and Indoor Herbs, Vegetables, Flowers and Succulent Plant | $64.39 | Amazon Link |
| 2 Pack of Plastic Plant Tray Saucer Rectangular 24 inch, Plant Trays Saucers for Indoors Outdoor, Black | $77.79 | Amazon Link |